NETWORK CHANGE MANAGEMENT

Complete Process, Request Form & Example

A practical document for Network Operations teams

| CHANGE LIFECYCLE | PURPOSE |
| --- | --- |
| 1. REQUEST | Describe the proposed change |
| 2. ASSESS | Understand risk, impact and dependencies |
| 3. APPROVE | Obtain the required authorization |
| 4. CHANGE | Perform the planned implementation |
| 5. VERIFY | Confirm the expected result |
| 6. ROLLBACK | Restore the previous state if required |

## Core idea

A controlled network change should be planned, approved, documented, implemented, verified and recoverable.

# 1. What Is Change Management?

Change management is the structured process used to control changes to network devices, services, configurations, cabling and infrastructure. The goal is to reduce unexpected outages and make it possible to understand what changed, why it changed and what happened afterward.

## Simple example

A technician needs to move Switch Port 5 from VLAN 10 (Users) to VLAN 20 (Voice). Instead of changing it immediately, the team records the request, checks the impact, gets approval, makes the change during the planned window, tests the result and keeps a rollback plan.

## What change management asks

☐ What is changing?    ☐ Why is it changing?    ☐ What could be affected?    ☐ Who approved it?    ☐ When will it happen?    ☐ How will we verify success?    ☐ How will we roll it back?

# 2. Common Network Changes

| Change type | Example | Typical risk |
| --- | --- | --- |
| Configuration | Change VLAN, IP, ACL or routing setting | Connectivity/security impact |
| Hardware | Replace switch, router, AP or firewall | Service outage |
| Software/Firmware | Upgrade network device firmware | Boot/compatibility risk |
| Cabling | Move or replace a network link | Loss of connectivity |
| Service | Add DNS, DHCP, VPN or monitoring service | Application/user impact |
| Security | Change firewall rule or access policy | Access or security impact |
| Capacity | Add bandwidth, APs or switch ports | Limited impact if planned |

# 3. Roles and Responsibilities

| Role | Responsibility |
| --- | --- |
| Requester | Explains the need and business reason |
| Network Engineer | Designs and performs the technical change |
| Change Approver | Reviews risk and authorizes the change |
| Service/Application Owner | Confirms application or business impact |
| Operations/Help Desk | Communicates and monitors user impact |
| Change Owner | Coordinates the complete change and records the outcome |

# 4. Change Assessment

Before approval, assess the change using a simple risk view.

| Factor | Questions to ask |
| --- | --- |
| Scope | Which devices, VLANs, sites or users are affected? |
| Impact | Could users, applications or security controls be interrupted? |
| Dependencies | Does another device/service depend on this configuration? |
| Timing | Is there a maintenance window? Is peak business time avoided? |
| Complexity | How many steps and devices are involved? |
| Recovery | Can the previous state be restored quickly? |
| Validation | What exact tests prove success? |

## Suggested risk rating

| Risk | Meaning | Example |
| --- | --- | --- |
| Low | Small scope; easy recovery | Change one unused switch port |
| Medium | Limited service impact possible | Change VLAN on an active user port |
| High | Major outage/security risk possible | Core routing or firewall change |
| Critical | Wide or business-critical impact | Data-center core redesign |

# 5. Network Change Request Form

Use this form for each planned change. Attach diagrams, configuration backups and test evidence where appropriate.

| Field | Entry |
| --- | --- |
| Change ID | CHG-________________ |
| Date requested | ________________ |
| Requester | ________________ |
| Change owner | ________________ |
| Business/service owner | ________________ |
| Planned date | ________________ |
| Maintenance window | ________________ |
| Priority | ☐ Normal ☐ Urgent ☐ Emergency |
| Risk | ☐ Low ☐ Medium ☐ High ☐ Critical |

## A. Change description

What is being changed?

________________________________________________________________________________

________________________________________________________________________________

Why is the change required?

________________________________________________________________________________

________________________________________________________________________________

Devices / sites affected?

________________________________________________________________________________

________________________________________________________________________________

Expected result?

________________________________________________________________________________

________________________________________________________________________________

## B. Current state / baseline

| Item | Current value / evidence |
| --- | --- |
| Device name(s) |  |
| Interface(s) |  |
| VLAN / subnet |  |
| IP address(es) |  |
| Current configuration backup |  |
| Current service status |  |

## C. Impact assessment

☐ No user impact expected    ☐ Planned service interruption    ☐ Security impact    ☐ Application dependency    ☐ External dependency

Potential impact / affected users:

________________________________________________________________________________

________________________________________________________________________________

## D. Implementation plan

| Step | Action | Owner | Expected result |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |

## E. Verification plan

| Test | Expected result | Actual result | Pass/Fail |
| --- | --- | --- | --- |
| Link/interface status |  |  |  |
| VLAN / routing test |  |  |  |
| Ping / connectivity |  |  |  |
| DNS / service test |  |  |  |
| Application/user test |  |  |  |

## F. Rollback plan

What exact steps restore the previous known-good state?

________________________________________________________________________________

________________________________________________________________________________

Rollback trigger: _________________________________________________________________

Rollback owner: __________________________________________________________________

# 6. Approval and Authorization

| Approval | Name | Date | Decision | Comments |
| --- | --- | --- | --- | --- |
| Change owner |  |  | ☐ Approved ☐ Rejected |  |
| Network approver |  |  | ☐ Approved ☐ Rejected |  |
| Service owner |  |  | ☐ Approved ☐ Rejected |  |
| Business owner (if required) |  |  | ☐ Approved ☐ Rejected |  |

Important: approval should occur before implementation except for changes handled under the organization's emergency-change process.

# 7. Implementation Record

| Field | Actual result |
| --- | --- |
| Start time |  |
| End time |  |
| Engineer |  |
| Devices changed |  |
| Commands/configuration applied |  |
| Backup taken before change | ☐ Yes ☐ No |
| Unexpected issue |  |

## Implementation steps completed

| Step | Completed | Notes / evidence |
| --- | --- | --- |
| 1 | ☐ |  |
| 2 | ☐ |  |
| 3 | ☐ |  |
| 4 | ☐ |  |
| 5 | ☐ |  |

# 8. Verification & Closure

☐ Interface/link is up    ☐ Correct VLAN/IP/routing state    ☐ Required services work    ☐ Users/application tested    ☐ Monitoring is normal    ☐ Documentation updated

Verification evidence / test results:

________________________________________________________________________________

________________________________________________________________________________

| Closure decision | Selection |
| --- | --- |
| Successful | ☐ Close change |
| Successful with issues | ☐ Close with follow-up |
| Failed and rolled back | ☐ Close after rollback |
| Still unresolved | ☐ Escalate / keep open |

# 9. Rollback Record

| Field | Entry |
| --- | --- |
| Rollback required? | ☐ Yes ☐ No |
| Reason / trigger |  |
| Time rollback started |  |
| Previous configuration restored | ☐ Yes ☐ No |
| Verification completed | ☐ Yes ☐ No |
| Service restored | ☐ Yes ☐ No |
| Follow-up required |  |

# 10. Emergency Change Process

Emergency changes are used when immediate action is required to restore service, address an active security issue or prevent significant business impact. Follow the organization's emergency approval process and document the change as soon as practical.

☐ Reason emergency action was required    ☐ Current impact recorded    ☐ Emergency approval obtained where possible    ☐ Implementation documented    ☐ Verification completed    ☐ Post-change review scheduled

# 11. Completed Example — Move Port 5 from VLAN 10 to VLAN 20

Scenario: A company IP phone is being installed at a desk. The switch port currently belongs to VLAN 10 (Users). The approved design requires the port to support VLAN 20 (Voice).

| Change field | Example entry |
| --- | --- |
| Change ID | CHG-2026-0147 |
| Device | SW-ACCESS-01 |
| Interface | Gi1/0/5 |
| Current state | VLAN 10 — Users |
| Target state | VLAN 20 — Voice |
| Reason | Deploy IP phone at Finance desk |
| Risk | Medium |
| Window | Saturday 22:00–22:30 |
| Expected result | Phone receives correct network access |

## Implementation

| Step | Action | Expected result |
| --- | --- | --- |
| 1 | Record current port configuration | Baseline saved |
| 2 | Apply approved VLAN/voice configuration | Port matches design |
| 3 | Connect IP phone | Phone link comes up |
| 4 | Check IP assignment | Phone receives expected address |
| 5 | Place test call / service test | Voice service works |

## Verification

☐ Port up    ☐ Correct VLAN    ☐ Expected IP/DHCP    ☐ Phone service works    ☐ Monitoring normal

Rollback: restore the previous port configuration and verify the original user connectivity.

# 12. Change Management vs Configuration Management

| Change Management | Configuration Management |
| --- | --- |
| Focuses on the change event | Focuses on configuration state |
| Was it planned and approved? | What configuration is active now? |
| Tracks implementation and verification | Tracks known-good/current state |
| Example: move Port 5 to VLAN 20 | Example: confirm Port 5 is actually in VLAN 20 |

Memory aid: Change Management asks “What changed?” Configuration Management asks “What is configured now?”

# 13. Network Change Checklist

☐ Request created    ☐ Business reason documented    ☐ Scope identified    ☐ Current state captured

☐ Impact assessed    ☐ Dependencies checked    ☐ Risk rated    ☐ Rollback prepared

☐ Approval obtained    ☐ Maintenance window confirmed    ☐ Stakeholders notified    ☐ Backup completed

☐ Change performed    ☐ Verification completed    ☐ Monitoring checked    ☐ Records updated

☐ Topology/inventory updated    ☐ Lessons learned recorded    ☐ Change closed

# 14. Trainer / Learner Quick Exercise

Scenario: A network engineer wants to change the default gateway for VLAN 30 (Guest) from 192.168.30.1 to 192.168.30.254.

Ask learners to identify:

☐ What should be recorded before the change?    ☐ Who could be affected?    ☐ What should be approved?    ☐ What tests prove success?    ☐ What is the rollback plan?

Expected thinking: baseline → impact → approval → implementation → verification → rollback.